Low Speed Flow Past a High Aspect Ratio Wing Lab

Pre-Lab Notes

You should be familiar with Prof. Buxton’s lecture Chapters 1, 2, 3, 5.1 and 5.2 before reading this note

Swept Wing Lecture

Infinite swept wing

Only 3D influences arise due to sweep alone

Bounded vortex in the section of the wing

- Delay of Mcrit in high-Mach-number subsonic flight, as the wing can remain subcritical

- Shock wave formation

- Flow becomes compressible

- Fly beyond Mcrit might reach MDD where wave drag increases dramatically

* Adding sweep also could result in thinner wing, which reduces ‘acceleration’ of flow on wing surface

- Velocity perturbation changes to uξ=u∞cosϕ and uη=u∞sinϕ

Streamline curved over the wing (non-zero pressure gradient being created)

* For wing of large AR, the conditions of infinite sweep almost meet at the half-wing

Finite aspect ratio

When AR≥4, we say it is large AR. When AR≤3, the effects at wing tip and root merge.

Platforms of minimum induced drag, constant lift and elliptical chord are no longer overlayed

– lifting line theory becomes invalid

Vortex pattern near the center of a sweptback wing - Shed Γat trailing edge

- At root

- Shed Γ at trailing edge - overall loss of lift

- Aerodynamic centre moves aft

- At tip

- Increase suction peak pressure gradient

- Aerodynamic centre moves forward

- Potential tip stall due to larger adverse pressure gradient (also because spanwise flow that pushes the boundary layer outboard) – results huge rolling moment

- Earlier sonic condition at tip than root due to addition of transverse flow

* Taper leading edge to overcome the last two issues

Pressure distribution over flat and swept back wing with the large aspect ratio

- Note how the effect of shed vorticity at the centre line is to push the pressure peak downstream, while the effect of the tip trailing vorticity is to push the pressure peak upstream.

As a result, the pressure drag of the swept wing is highest at the centre.

Effect of leading edge tapered swept wing

- Uniform taper

- Reduced lift at the tip - smaller chord at the tip

- Reduced risk of tip stall - aerodynamic load is shifted more towards the root

- More elliptical lift distribution

- Counteract the adverse effects of spanwise flow

- Linear Twist (Washout)

- The root section of the wing will reach its stall angle before the tip - reducing the effective angle of attack at the wingtip compared to the root

- Potential tip stall due to larger adverse pressure gradient (also because the spanwise flow that pushes the boundary layer outboard) – results in a huge rolling moment

- Counteract the adverse effects of spanwise flow

*Will tip stall first in our experiment?

Surface Source Singularity (Panel Code)

Method

Assume incompressible, potential (inviscid, irrotational) flow

Divide the surface with various points connected by ‘panel’ with source density m

– One of point must be the trailing edge if body is airfoil

Body coordinate system

- Find velocity components at the control point of the pth element due to the qth element

- Find vxq and vyq as function of mq

- Find normal velocity due to the qth element at pth element vnp, q=Ap, qmq

- Find tangential velocity due to the qth elemen at pth element vtp, q=Bp, qmq

- Find velocity components at pth element due to free stream and AoA

- Find free stream normal velocity at pth element Unp=U∞sinα-θp

- Find free stream tangential velocity at pth element Utp=U∞cosα-θp

- Compute source density at pth element

- Normal velocity at pth element q=1Nvnp, q+Unp=0 (linearized boundary condition)

- Compute tangential velocity components at pth element

- Tangential velocity at pth element Qp=q=1Nvtp, q+Utp

- Find additional source density due to the application of the unit circulation Γ=U∞c

- Normal velocity at pth element q=1NvnΓp, q=UnΓp

- Find number k of unit circulation that fulfills Kutta condition at the trailing edge

- Kutta condition is QT+QT+1=∆Qt.e=0

- Therefore, k∆QΓt.e+∆Qt.e=0

- Find the surface velocity at pth element by superposition

- Normal velocity at pth element Qp=Qp+kQpΓ

- Pressure coefficient at pth element Cpp=1-Qp2U∞2

- Find lift coefficient

- Life coefficient CL=2k

Pros and cons of using panel code estimate swept wing aerodynamics

- Linear boundary problems can be computed when viscous effects are either unimportant or can be dealt with in a simpler way such as by the addition of a circulation

Pros:

- Simplicity

- Faster Computation by avoiding the computational complexity of full Navier-Stokes or boundary layer models

Cons:

- Cannot accurately capture flow separation or stall behaviour due to boundary layer detachment

- Poor drag estimation - pressure drag and skin friction drag are not directly accounted for by inviscid panel codes

- Complex flow interaction on the swept wing, involving spanwise flow, adverse pressure gradients, and tip vortices, panel code cannot fully capture the complex interplay of 3D viscous forces

- Lifting problem in 3D

Cons:

- Very lengthy iteration processes are needed to obtain the correct position for shed trailing vortex wake

* Test the accuracy of the panel code by changing the number of elements around the airfoil (A.M.O Smith suggests 3D bodies 500-1000 panels)

** Curved elements can be used for 3D bodies for more algebra but fewer elements

cd "C:\Users\GH-X-ST\OneDrive - Imperial College London\Year 3\Aerodynamics 3\A - Low Speed Flow Past a High Aspect Ratio Wing Lab\01 - Lab Files\panel_code"

Measure-Command { .\combo.exe mesh.dat auxiliary.dat stream.dat sect inp }

Effect of resolution

- Chord-wise discretisation

- Finer discretization leading a more accurate lift calculation, as described the shape of the airfoil

- Span-wise discretisation

- Diverging occurs at the wing tip when the number of span-wise discretisation is smaller or equal to the number of section ( 3 section in this experiment)

Experiment

Procedure

- Read Pressure

- Set wing to α=0°

- Record manometer readings to data.xlsx

- Repeat by set wing to α=6° and α=13°

*Change AoA gradually to avoid the stepper motor stall (when the motor does not have enough torque to overcome the load when accelerating/decelerating too fast)

- Stall the wing

- Set wing back to α=0°

- Increase incidence

- Record incidence at a stall by observing the wool tufts and pressure distribution

- Make sketches of the qualitative shape of the pressure distribution for each of section

*Wait some time to make stall progress throughout the wing before recording

**Try to record different threshold value of AoA for tip and root stall

*** Stall angle for infinite wing in [3] should be 17 degrees

- Wool tufts

- Post stall - lie smoothly along the surface of the wing while pointing slightly outboard due to the spanwise component of flow

- Tip Stall (which is expected for swept wing) Initiation - wool tufts near the wingtip will flutter and oscillate

- Separation progresses inboard

- Full Stall - wool tufts across most or all of the wing will be flapping wildly

*Record videos of the wool tufts at above experiments and add screenshots to the PowerPoint

- Pressure distribution at different AoA

Expected pressure distribution at different AoA

Conclusion

- The influence of sweep and infinite aspect ratio on the sectional lift-coefficient slope of a swept wing, is panel code a good approximation?

- Provide plots in MATLAB for swept wing

- Record manometer readings to data.xlsx

- Repeat by set wing to α=6° and α=13°

*Change AoA gradually to avoid the stepper motor stall (when the motor does not have enough torque to overcome the load when accelerating/decelerating too fast)

- Stall the wing

- Set wing back to α=0°

- Increase incidence

- Record incidence at a stall by observing the wool tufts and pressure distribution

- Make sketches of the qualitative shape of the pressure distribution for each of section

*Wait some time to make stall progress throughout the wing before recording

**Try to record different threshold value of AoA for tip and root stall

*** Stall angle for infinite wing in [3] should be 17 degrees